Data Communication

(CAECC12)

Data Communication

(CAECC12)

Lab Practicals

Lab Practicals

Nupur Sudhakar

2019UCS2048

CSAI

Nupur Sudhakar

2019UCS2048

CSAI

INDEX

INDEX

| S.No. | Topic | Page No. |
| --- | --- | --- |
| 1 | Introduction |  |
|  | - To perform matrix computation | 1 – 2 |
|  | - To plot sine wave of frequency 200 Hz. | 3 – 4 |
|  | - To plot a pulse of width 10. | 5 – 6 |
|  | - To plot the spectrum (amplitude and phase) of the pulse generated above. | 7 – 9 |
| 2 | To generate uniform distributed random number and plot its density function. Also find its mean and variance. | 10 – 11 |
| 3 | To generate Gaussian distributed random number and plot its density function. Also find its mean and variance. | 12 – 13 |
| 4 | Compute the signal to quantisation noise ratio of Uniform Quantisation. Plot SNQR versus Quantisation levels. | 14 – 16 |
| 5 | Study of passband digital communication technique BPSK. Calculate the BER of BPSK modulated signal. | 17 – 19 |
| 6 | Given is a linear block code with the generator matrix G :- 1 1 0 0 1 0 1 G = 0 1 1 1 1 0 0 1 1 1 0 0 1 1 - Calculate the number of valid code words N and the code rate RC. Specify the complete Code set C. - Determine the generator matrix G′ of the appropriate systematic (separable) code C’. - Determine the syndrome table for single error. | 20 – 24 |

Practical - 1

a)

Aim

To perform matrix computation

Theory

A matrix is a rectangular array or table of numbers arranged in rows and columns. We can perform various operations on matrices like addition, subtraction multiplication, inverse, transpose etc.

Software used

Octave

Code & Output

Result

We have successfully performed matrix computation.

b)

Aim

To plot sine wave of frequency 200 Hz.

Theory

The sine wave is the most fundamental form of a periodic analog signal. When we visualize it as a simple oscillating curve, its change over the course of a cycle is smooth and consistent, a continuous, rolling flow. Each cycle consists of a single arc above the time axis followed by a single arc below it.  A sine wave can be represented by three parameters: the peak amplitude, the frequency and the phase.

Software used

Octave

Code

t=[0:0.0001:0.005];

f=200;

x=sin(2*pi*f*t);

plot(t,x);

xlabel("time");

ylabel("sin(x)");

Output

Result

A sine wave of frequency 200 Hz has been successfully plotted.

c)

Aim

To plot a pulse of width 10.

Theory

A pulse in signal processing is a rapid, transient change in the amplitude of a signal from a baseline value to a higher or lower value, followed by a rapid return to the baseline value.  The pulse width is a measure of the elapsed time between the leading and trailing edges of a single pulse of energy.

Software used

Octave

Code

x=[-10:0.005:10];

y=sign(x+5)-sign(x-5);

z=y*0.5;

z=z*0.2;

plot(x,z);

xlabel("time");

ylabel("unit pulse");

Output

Result

A pulse of width 10 has been successfully plotted.

d)

Aim

To plot the spectrum (amplitude and phase) of the pulse generated above.

Theory

The amplitude spectrum simply gives amplitude at each frequency. The phase spectrum simply gives the phase at each frequency.

Software used

Octave

Code

- Amplitude spectrum

x=[-10:0.005:10];

y=sign(x+5)-sign(x-5);

z=y*0.5;

z=z*0.2;

k=fft(z);

c=fftshift(k);

plot(x,c);

axis([-0.15,0.15]);

xlabel("frequency");

ylabel("amplitude spectrum");

- Phase spectrum

x=[-10:0.005:10];

y=sign(x+5)-sign(x-5);

z=y*0.5;

z=z*0.2;

k=fft(z);

c=fftshift(k);

g=angle(c);

plot(x,g);

axis([-0.15,0.15]);

xlabel("frequency");

ylabel("phase spectrum");

Output

- Amplitude spectrum

- Phase spectrum

Result

Amplitude and phase spectrums of the pulse have been successfully plotted.

Practical - 2

Aim

To generate uniform distributed random number and plot its density function. Also find its mean and variance.

Theory

A uniform distribution, also called a rectangular distribution, is a probability distribution that has constant probability.

The expected value (i.e. the mean) of a uniform random variable X is:

E(X) = (1/2).(a + b)

The variance of a uniform random variable is:

Var(x) = (1/12).(b-a)2

Software used

Octave

Code

N = 100000

x = rand(1,N);

hist(x)

m = mean(x)

v = var(x)

Output

Result

Uniform distributed random number has been generated and its density function has been successfully plotted.

Practical - 3

Aim

To generate Gaussian distributed random number and plot its density function. Also find its mean and variance.

Theory

A Gaussian (or normal) distribution is an arrangement of a data set in which most values cluster in the middle of the range and the rest taper off symmetrically toward either extreme.

A graphical representation of a normal distribution is sometimes called a bell curve because of its flared shape. The precise shape can vary according to the distribution of the population but the peak is always in the middle and the curve is always symmetrical. In a Gaussian distribution, the mean, mode and median are all the same.

The general form of its probability density function is –

{\displaystyle f(x)={\frac {1}{\sigma {\sqrt {2\pi }}}}e^{-{\frac {1}{2}}\left({\frac {x-\mu }{\sigma }}\right)^{2}}}

The parameter {\displaystyle \mu }μ is the mean or expectation of the distribution while the parameter {\displaystyle \sigma }σ is its standard deviation. The variance of the distribution is σ2.

Software used

Octave

Code

N = 100000

x = randn(1,N);

hist(x)

m = mean(x)

v = var(x)

Output

Result

Gaussian distributed random number has been generated and its density function has been successfully plotted.

Practical - 4

Aim

Compute the signal to quantisation noise ratio of Uniform Quantisation. Plot SNQR versus Quantisation levels.

Theory

The digitization of analog signals involves the rounding off of the values which are approximately equal to the analog values. The method of sampling chooses a few points on the analog signal and then these points are joined to round off the value to a near stabilized value. Such a process is called as Quantization.

The type of quantization in which the quantization levels are uniformly spaced is termed as a Uniform Quantization.

SQNR, short for signal to quantization noise ratio, is a measure of the quality of the quantisation and or digital conversion of an analog signal.

Software used

Octave

Code

levels = 1:1000;

amplitude = 5;

frequency = 1;

time = 0:0.001:1;

x = amplitude*sin(2*pi*frequency*time);

mx = max(x);

mn = min(x);

step = (mx-mn)./levels;

for i=levels

in = floor((x-mn)/step(i));

xq = mn+ in*step(i);

noise = xq - x;

rmsofnoise = var(noise);

power = amplitude*amplitude/2;

sqnr(i) = power/rmsofnoise;

end

sqnrPractical = sqnr;

plot(levels, sqnrPractical);

grid on;

Output

Result

The signal to quantisation noise ratio of Uniform Quantisation has been computed and SNQR versus Quantisation levels have been plotted.

Practical - 5

Aim

Study of passband digital communication technique BPSK. Calculate the BER of BPSK modulated signal.

Theory

In any phase modulation scheme the information is expressed in terms of phase of the carrier. Phase of the carrier signal is shifted according to the input binary data. BPSK (Binary Phase Shift Keying) is the simplest form of phase shift keying (PSK). In BPSK, individual data bits are used to control the phase of the carrier. During each bit interval, the modulator shifts the carrier to one of two possible phases, which are 180 degrees or π radians apart. Each bit of the digital signal produces a transmit symbol with duration Ts, which is equal to the bit duration Tb.

Bit error rate (BER) of a communication system is defined as the ratio of number of error bits and total number of bits transmitted during a specific period. It is the likelihood that a single error bit will occur within received bits, independent of rate of transmission.

Software used

Octave

Code

x = randi(2, 1, 1000);

x(x == 2) = -1;

snr =[-40:4:40];

ber = zeros(1, length(snr));

for i = 1:length(snr)

y = awgn(x, snr(i));

for j = 1:length(x)

if x(j) > 0 && y(j) <= 0

ber(i) = ber(i) + 1;

endif

if x(j) <= 0 && y(j) > 0

ber(i) = ber(i) + 1;

endif

endfor

endfor

disp(ber);

figure;

plot(snr, ber);

xlabel('SNR');

ylabel('Bit error rate');

title('BER Simulation for BPSK');

Output

Result

The BER of BPSK modulated signal has been calculated.

Practical - 6

Aim

Given is a linear block code with the generator matrix G :-

1 1 0 0 1 0 1

G = 0 1 1 1 1 0 0

1 1 1 0 0 1 1

- Calculate the number of valid code words N and the code rate RC. Specify the complete Code set C.

- Determine the generator matrix G′ of the appropriate systematic (separable) code C’.

- Determine the syndrome table for single error.

Theory

In coding theory, a generator matrix is a matrix whose rows form a basis for a linear code. The codewords are all of the linear combinations of the rows of this matrix, that is, the linear code is the row space of its generator matrix.

Software used

Octave

Code

a)

% Generator Matrix

G = [

1 1 0 0 1 0 1;

0 1 1 1 1 0 0;

1 1 1 0 0 1 1;

];

N = 7; % total bits

K = 3; % no of message bits

fprintf("Code rate is %d\n", K/N);

possibleCodes = [

0 0 0;

0 0 1;

0 1 0;

0 1 1;

1 0 0;

1 0 1;

1 1 0;

1 1 1;

];

codeSet = [];

for i=1:8

code = possibleCodes(i,:);

codeSet = [codeSet;mod(code*G,2)];

endfor

fprintf("\nCode set : \n");

disp(codeSet)

b)

G = [ 1 1 0 0 1 0 1;

0 1 1 1 1 0 0;

1 1 1 0 0 1 1;

];

K=3;

N=7;

% Generating appropriate systematic code

G(3,:) = mod(G(3,:) + G(1,:),2);

G(2,:) = mod(G(3,:) + G(2,:),2);

G(1,:) = mod(G(2,:) + G(1,:),2);

fprintf("Generator matrix :\n");

disp(G)

possibleCodes = [

0 0 0;

0 0 1;

0 1 0;

0 1 1;

1 0 0;

1 0 1;

1 1 0;

1 1 1;

];

codeSet = [];

for i=1:8

code = possibleCodes(i,:);

codeSet = [codeSet;mod(code*G,2)];

endfor

fprintf("\nCode set :\n");

disp(codeSet)

c)

% Generator Matrix

G = [

1 1 0 0 1 0 1;

0 1 1 1 1 0 0;

1 1 1 0 0 1 1;

];

N = 7;

K = 3;

H = G';

r = [0 0 0 1 0 1 0];

p = mod(r*H,2);

num=0;

pos=-1;

p

for i=1:7

if isequal(p,H(i,:))

pos=i;

break;

endif

endfor

fprintf("Error in %d\n",pos);

r(pos) = mod(r(pos)+1,2);

disp(r)

Output

a)

b)

c)

Result

All the operations have been performed and the calculated values are same as expected values.